| Role | Name | Date |
| --- | --- | --- |
| Document Owner: | Mike Dawson | 5/17/2026 |
| Document Concurrence: |  |  |
| Management Approval: |  |  |

| Rev | Notes | ECN |
| --- | --- | --- |
| 1 | Initial Draft | N/A |

Purpose and Scope

The purpose of this document is to outline the testing steps to validate the functional status of the ESP32 DRV8825 Stepper Driver Development Board. Only applies to PCB assemblies that are designated to be sold to the public and to determine proper operation prior to shipping.

Required Materials

- Completed PCB assembly for the P1012202501 project including all accessories and enclosure hardware as specified for the product BOM

- Current revision of this document for production test process

- Current copy of the “P1012202501-3 Production Test Log Template.xlsx” to store results

- Serial number labels for the PCB assemblies, follow the work instruction *****

- Benchtop power supply capable of sourcing 24VDC at 3A.

- Power supply test cable # P1012202501-PS-TEST

- Digital muti-meter capable of measuring resistance and voltage

- Benchtop test fixture for the PCB assembly

- PC with access to current production firmware and the ESP32 flashtool

- USB-C interface cable that supports data transfer

- USB to RS232 FTDI adapter cable

- Prewired USB to RS485 adapter cable assembly # P1012202501-RS485-TEST

- Any documentation and packaging required to prepare the final assembly for shipping

Prior to Testing

Open the “P1012202501-3 Production Test Log Template.xlsx” file and save a copy into the test-logs directory while changing the name to “P1012202501-3 Production Test Log MOXXXXXX.xlsx” where the MOXXXXXX stands for Manufacturing Order then the six digit number of the order itself such as MO00022. This will allow traceability back to the MRP system. Enter the relevant data for the equipment used in the testing process such as the power supply and digital multimeter.

Each line is for a separate PCB assembly and must be filled out completely for proper traceability. Failed PCB assemblies must be properly marked and quarantined from the final shipping goods. Refer to SOP XXXX for further information. The production test log shall indicate the status of all PCB assemblies that have been tested.

Any failed PCB assemblies shall be evaluated by engineering for the cause of the failure and identified issues shall be elevated as necessary in the case of a systemic issue. Refer to SOP XXXX for further information.

Connect P1012202501-PS-TEST cable to the benchtop power supply with the RED wire connecting to the POSITIVE (+) port and the BLACK wire connecting to the NEGATIVE (-) port on the power supply.

Here are the steps to properly load up and configure the ESP32 flash tool software.

Test Section 1 – Visual Inspection

TS1-0001:

Visually inspect the PCB assembly for proper workmanship, following guidelines for IPC Class II assemblies. Visually inspect any enclosure hardware for problems such as breakages and cracks. Verify all materials are present and accounted for. Enter pass or fail in the test log for this section.

Test Section 2 – Prior to Powerup

TS2-0001:

Using the multi-meter in resistance mode, check between TP1 (+3.3V) and TP3 (GND) and verify that the resistance is greater than 5KΩ, if so, please enter PASS for the result.

TS2-0002:

Using the multi-meter in resistance mode, check between TP2 (+5.0V) and TP3 (GND) and verify that the resistance is greater than 5KΩ, if so, please enter PASS for the result.

Test Section 3 – Powerup

TS3-0001:

Verify that the power supply is set to 24VDC and the current limit is set to 3A and the output is turned off. Connect the first DUT (Device Under Test) to the power connector. Enable the output of the power supply and observe if the 3.3V (D1) and the 5.0V (D2) LEDs become active. Enter the result into the log file.

TS3-0002:

With the power supply output enabled, configure a multimeter to DC voltage measurement and connect the negative lead to TP3 (GND) and the positive lead to TP1 (3.3V). Check and verify that the voltage output is positive 3.3V +/- 125mV. Record the result in the test log.

TS3-0003:

With the power supply output enabled, configure a multimeter to DC voltage measurement and connect the negative lead to TP3 (GND) and the positive lead to TP2 (5.0V). Check and verify that the voltage output is positive 5.0V +/- 125mV. Disable the power supply output. Record the result in the test log.